This repository includes four main files to perform current deconvolution analysis for cyclic voltammetry (CV) data. The goal is to quantify how different current contributions change with potential and scan rate, and to provide clear plots and outputs for interpretation.

deConV_MATLAB_GUI.mlapp:

It is the main MATLAB application (built with MATLAB App Designer). You open this app to run the deConV analysis in MATLAB. It is designed to import CV data in the correct format, perform the fitting/deconvolution procedure, calculate the contribution of each modeled mechanism across the potential window, and visualize the results.

deConV.ipynb:

It provides the same general workflow as the MATLAB app, but in a notebook format that is easy to run step-by-step and reproduce. The notebook is typically used to load CV datasets, preprocess or align data (for example, making scan rates consistent and organizing current vs. potential), apply the deconvolution model, and generate plots and tables.

FileFormatter.mlapp:

It is a MATLAB helper app used for data preparation. Its main purpose is to compile multiple scan-rate CV files into one combined file that is suitable for importing into the MATLAB deConV app. Many users store CV data as separate files for each scan rate (e.g., 10 mV/s, 20 mV/s, 50 mV/s, etc.). The FileFormatter app collects those files, arranges them in a consistent structure, and outputs a single formatted dataset so the MATLAB GUI can read it smoothly. It is important to note that you do not necessarily need to install this app separately, because it is already included in the toolbox installer described below.

deConV_MATLAB_Package_Installer.mltbx:

It is a MATLAB toolbox/package installer. This is the recommended installation method because it installs the MATLAB apps (including the FileFormatter) in the standard MATLAB way and makes them available in your MATLAB environment. To install it, download the .mltbx file, then open MATLAB and go to Home → Add-Ons → Install from File…, select the installer file, and follow the prompts. After installation, the apps will appear in the MATLAB Apps tab (or in your installed add-ons), and you can launch them directly from there.

To gain further guidance on interpreting the results, troubleshooting any errors that may occur, and using the files correctly, we strongly recommend referring the associated paper and its Supporting Information.

https://doi.org/10.1016/j.electacta.2026.148127